# DHCP Failure & Recovery – Full Technical Case Study

## 1. Purpose of This Document

This document is a complete, real‑world troubleshooting record of DHCP failures encountered in a multi‑layer campus network built in Cisco Packet Tracer, and how those failures were systematically identified, isolated, and fixed.

## 2. Topology Overview (What Was Built)

### 2.1 Network Design Model

- Hierarchical campus design

  - Core Layer (L3, routing, HSRP)

  - Distribution Layer (VLAN segmentation, STP, DHCP relay)

  - Access Layer (end devices, security controls)

### 2.2 Devices Used

| Layer | Device Type | Model | Hostname |
| --- | --- | --- | --- |
| Core | Multilayer Switch | 3560‑24PS | Core‑SW‑1 |
| Core | Multilayer Switch | 3560‑24PS | Core‑SW‑2 |
| Distribution | Switch | 2960‑24TT | Dist‑Pharm |
| Distribution | Switch | 2960‑24TT | Dist‑Admin |
| Access | Switch | 2960‑24TT | Acc‑Pharm |
| Access | Switch | 2960‑24TT | Acc‑Admin |
| Services | Server | Server‑PT | Server0 (DHCP/DNS/NTP) |
| Endpoints | PCs | PC‑PT | PC0–PC7 |
| Endpoints | Laptops | Laptop‑PT | Laptop0–Laptop4 |
| Wireless | AP | AccessPoint‑PT | AP‑Pharm / AP‑Admin |

## 3. VLAN & IP Addressing Plan

| VLAN | Department | Subnet | Gateway (HSRP) |
| --- | --- | --- | --- |
| 20 | Emergency | 10.10.20.0/24 | 10.10.20.1 |
| 30 | Radiology | 10.10.30.0/24 | 10.10.30.1 |
| 40 | Ward | 10.10.40.0/24 | 10.10.40.1 |
| 50 | Pharmacy | 10.10.50.0/24 | 10.10.50.1 |
| 60 | Admin | 10.10.60.0/24 | 10.10.60.1 |
| 90 | Servers | 10.10.90.0/24 | 10.10.90.1 |
| 99 | Native / Mgmt | 10.10.99.0/24 | N/A |

## 4. Physical & Logical Connectivity

### 4.1 Cabling Used

- Copper straight‑through

  - PC ↔ Access Switch

  - AP ↔ Access Switch

  - Server ↔ Core Switch

- Copper crossover / Auto‑MDIX

  - Switch ↔ Switch (Access ↔ Distribution, Distribution ↔ Core)

### 4.2 Key Trunk Links

| From | Port | To | Port | Mode | Native VLAN | Allowed VLANs |
| --- | --- | --- | --- | --- | --- | --- |
| Acc‑Pharm | Fa0/24 | Dist‑Pharm | Fa0/1 | Trunk | 99 | 50,99 |
| Dist‑Pharm | Gi0/1 | Core‑SW‑1 | Gi0/1 | Trunk | 99 | All |
| Dist‑Pharm | Gi0/2 | Core‑SW‑2 | Gi0/1 | Trunk | 99 | All |

## 5. DHCP Architecture

### 5.1 DHCP Server

- Device: Server0

- Interface: FastEthernet0

- IP: 10.10.90.10

- Services:

  - DHCP

  - DNS (10.10.90.11)

### 5.2 DHCP Pools (Example)

| Pool | Gateway | Start IP | Mask |
| --- | --- | --- | --- |
| VLAN50 | 10.10.50.1 | 10.10.50.10 | /24 |
| VLAN60 | 10.10.60.1 | 10.10.60.10 | /24 |

## 6. Initial Problem Statement

### Symptoms Observed

- VLAN20/30/40/60 clients received DHCP successfully

- VLAN50 clients failed DHCP consistently

- Static IP worked

- Ping worked once static IP was applied

- DHCP request failed from:

  - Wireless laptops

  - Wired PCs

This proved not an AP issue.

## 7. Troubleshooting Timeline (Step‑by‑Step)

### Step 1 – Verify VLAN Presence

show vlan brief

✔ VLAN50 present on Access & Distribution switches

### Step 2 – Verify Trunking

show interfaces trunk

✔ VLAN50 allowed and forwarding

### Step 3 – Verify STP

show spanning-tree vlan 50

✔ No blocking on access paths

### Step 4 – Verify Gateway & Routing

show standby brief
ping 10.10.50.1

✔ HSRP Active ✔ Gateway reachable

### Step 5 – Verify DHCP Server Reachability

ping 10.10.90.10

✔ Reachable from Core

### Step 6 – Check DHCP Snooping (ROOT CAUSE)

show ip dhcp snooping

Critical Discovery: - DHCP Snooping enabled on VLAN50 - Uplink interfaces NOT trusted - DHCP replies dropped silently

## 8. Root Cause Analysis

### Why DHCP Failed

- DHCP Discover from client reached server

- DHCP Offer returned from server

- DHCP Snooping dropped Offer/Ack on untrusted uplinks

ICMP works → DHCP fails → classic DHCP Snooping misconfiguration

## 9. Fix Applied (Exact Commands)

### 9.1 Distribution Switch – Dist‑Pharm

conf t
interface gi0/1
 ip dhcp snooping trust
interface gi0/2
 ip dhcp snooping trust
end
wr

### 9.2 Access Switch – Acc‑Pharm

conf t
interface fa0/24
 ip dhcp snooping trust
end
wr

## 10. Verification After Fix

### DHCP Snooping Status

show ip dhcp snooping

✔ Uplinks trusted

### Client Renewal

ipconfig /release
ipconfig /renew

✔ IP assigned successfully

### Binding Table

show ip dhcp snooping binding

✔ Client MAC ↔ IP mapped

## 11. Final Outcome

- VLAN50 DHCP fully operational

- Wired & Wireless clients working

- Security preserved (Snooping kept enabled)

- No unnecessary feature removal

## 12. Key Lessons Learned

- DHCP failures are often security‑related, not routing issues

- Ping success ≠ DHCP success

- DHCP Snooping requires explicit trust on uplinks

- Multi‑layer networks must be debugged hop by hop

- Real networks fail in combinations, not single misconfigs

End of Document