# 🏥 Hospital Campus Network – Access Layer Configuration

## 📌 Overview

This repository documents the Access Layer configuration and hardening performed in a multi-department hospital campus network designed and implemented using Cisco Packet Tracer.

Unlike isolated CCNA-style labs, this project intentionally integrates multiple networking concepts together to reflect how real enterprise networks behave — including misconfigurations, troubleshooting, and iterative fixes.

## 🎯 Objectives

- Implement proper VLAN-based segmentation per department

- Configure secure access ports for end devices

- Establish stable trunk uplinks to Distribution switches

- Prevent Layer 2 loops using Spanning Tree Protocol (STP)

- Apply basic access-layer security controls

- Troubleshoot and resolve real-world configuration errors

## 🧱 Access Switches Covered

- Acc-Emerg – Emergency Department

- Acc-Rad-1 / Acc-Rad-2 – Radiology

- Acc-Ward-1 / Acc-Ward-2 – Wards

- Acc-Pharm – Pharmacy

- Acc-Admin – Administration

Each access switch connects upstream to its respective Distribution switch using a single 802.1Q trunk.

## 🧩 VLAN Design (Access Layer)

| VLAN ID | Name | Purpose |
| --- | --- | --- |
| 20 | EMERGENCY | Emergency Department |
| 30 | RADIOLOGY | Radiology |
| 40 | WARDS | Patient Wards |
| 50 | PHARMACY | Pharmacy |
| 60 | ADMIN | Administration |
| 70 | GUEST_WIFI | Guest Wireless |
| 80 | VOIP | IP Phones |
| 99 | NATIVE | Native / Infrastructure |

Only required VLANs were created on each access switch to avoid unnecessary VLAN propagation.

## ⚙️ Configuration Steps

### 1️⃣ VLAN Creation (Per Access Switch)

vlan <DEPARTMENT_VLAN>
vlan 70
vlan 80
vlan 99

### 2️⃣ Access Port Configuration & Hardening

interface range fa0/1 - 20
 switchport mode access
 switchport access vlan <DEPARTMENT_VLAN>
 spanning-tree portfast
 spanning-tree bpduguard enable
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security aging time 5
 ip dhcp snooping limit rate 10

Why this matters: - Prevents rogue switches (BPDU Guard) - Limits MAC flooding attacks - Protects against rogue DHCP servers - Enables fast host connectivity

### 3️⃣ Uplink (Trunk) Configuration

interface fa0/24
 switchport mode trunk
 switchport trunk native vlan 99
 switchport trunk allowed vlan <DEPARTMENT>,70,80,99
 ip dhcp snooping trust
 no shutdown

Key design choices: - Native VLAN 99 used consistently - Only required VLANs allowed on trunks - DHCP Snooping trust enabled only on uplinks

### 4️⃣ DHCP Snooping (Global)

ip dhcp snooping
ip dhcp snooping vlan <DEPARTMENT>,70,80
no ip dhcp snooping information option

## 🔍 Verification Commands Used

show interfaces trunk
show interfaces fa0/24 switchport
show vlan brief
show spanning-tree vlan <ID>
show cdp neighbors
show port-security
show ip dhcp snooping

Verification was always performed from both access and distribution switches.

## ❌ Problems Encountered & Resolutions

### 🔴 Native VLAN Mismatch

Error:

%CDP-4-NATIVE_VLAN_MISMATCH

Cause: - One side using VLAN 1 - Other side using VLAN 99

Fix:

switchport trunk native vlan 99

Configured on both ends of the link.

### 🔴 STP Showing No Forwarding VLANs

Causes: - VLAN not created on access switch - VLAN inactive or pruned - STP convergence delay

Resolution: - Manually created VLANs - Re-verified trunk allowed VLANs - Checked STP state after convergence

### 🔴 PortFast Warnings on Trunks

Message:

Portfast will only have effect when the interface is in non-trunking mode

Explanation: - PortFast was applied via interface range - Trunk ports correctly ignore PortFast

Action: - No change required

## ⚠️ Cisco Packet Tracer Limitations

| Limitation | Impact |
| --- | --- |
| Slow STP convergence | Temporary misleading outputs |
| Partial Port Security support | Some aging modes unavailable |
| DHCP Snooping inconsistencies | Required manual verification |
| Missing CLI commands | Used alternative show commands |

These limitations encouraged real troubleshooting habits instead of blind command trust.

## 🧠 Key Lessons Learned

- Enterprise networks are non-linear and interconnected

- Fixing one issue can expose another

- Always verify from both sides of the link

- STP behavior becomes intuitive with practice

- Access-layer security must be intentional

## ✅ Outcome

✔ Secure and stable access layer ✔ Proper VLAN segmentation ✔ Loop-free Layer 2 topology ✔ Hardened user-facing ports ✔ Real-world troubleshooting experience

## 💼 Portfolio Value

This project demonstrates: - Practical Layer 2 troubleshooting - Enterprise-style switch configuration - Security-aware access design - Understanding beyond certification labs

Note: This project was built for learning and portfolio purposes using Cisco Packet Tracer. While some platform limitations exist, all configurations follow real Cisco IOS design principles.